APB Results and Debugging- Prerna Agarwal

Simulation Result:

1st  RUN

Functional Coverage Analysis

| Metric | Covered | Total | % Hit | Status |
| --- | --- | --- | --- | --- |
| Overall Covergroup | 15 | 18 | 83.33% | ❌ Uncovered |

| Coverpoint Cp_write |  |  | Coverpoint cp_data |  |  |
| --- | --- | --- | --- | --- | --- |
| Bin | Hits | Status | BIn | Hits | Status |
| READ | 21 | ✅ Covered | Zero | 21 | ✅ Covered |
| WRITE | 19 | ✅ Covered | others | 19 | ✅ Covered |
| Coverpoints cp_addr |  |  | Coverpoint cp_error |  |  |
| Bin | Hits | status | Bin | Hits |  |
| Low (0-15) | 13 | ✅ Covered | no_error | 40 | ✅ Covered |
| Mid (16-127) | 10 | ✅ Covered | error | 0 | ❌ Missing |
| High(128-255) | 17 | ✅ Covered |  |  |  |

Cross Coverage - cp_write x cp_addr

| Combination | Hits | Status |
| --- | --- | --- |
| READ, low | 7 | ✅ Covered |
| WRITE, low | 6 | ✅ Covered |
| READ, mid | 4 | ✅ Covered |
| WRITE, mid | 6 | ✅ Covered |
| READ, high | 10 | ✅ Covered |
| WRITE, high | 7 | ✅ Covered |

Cross coverage = cp_write x cp_error

| Combination | Hits | Status |
| --- | --- | --- |
| READ, no_error | 21 | ✅ Covered |
| WRITE, no_error | 19 | ✅ Covered |
| READ, error | 0 | ❌ Missing |
| WRITE, error | 0 | ❌ Missing |

Problems :

- The current Environment has not generated PSLVERR = 1 means NO ERROR Transaction occurred.

- Reading to uninitialized Address (showing Warnings/Error)

- Error scenarios are not covered completely. hence ,Coverage results 83%

Approaches to Solve Problems:

- Generate Error Scenarios from DUT or sequence

| - Modify Slave if( PADDR > max_valid_addr) PSLVERR = 1; | - Force invalid access in sequence tr.PADDR = ‘hFFFF; //invalid addr Or Add constraint Constraint invalid_addr{ PADDR inside {[256:300]}; } |
| --- | --- |

So I  added this constraint in transaction class , Now DUT(slave) must implement this Behaviour to Flag this invalid access. Because COVERAGE can be close to 100% if DUT actually produces that behavior.

2)   Fixing -  “reading to uninitialized address”

Intuition- disable address randomization while reading to only test addresses which       are written.

❌ Weak Verificaation Quality

❌ Lose corner case testing

❌ Never hit Error(PSLVERR)

❌ Testing to written addres only -leads to incomplete behaviour

Actually we need to decide how our DUT is supposed to behave then model and test it

| - Return 0/x (PSLVERR = 0) Current my DUT is behaving this | - Signal error PSLVERR=1 meaning Data may be invalid if reading to uninitialized address occurs |
| --- | --- |

Solution applied :

In DUT:  should define specification

module apb_slave #(parameter int MAX_VALID_ADDR = 255) if(PWRITE)begin if( PADDR >= MAX_VALID_ADDR)begin PSLVERR <= 1; //master tired to access invalid address end else begin mem[PADDR[2:0]] <= PWDATA; //write end end else begin if(PADDR >= MAX_VALID_ADDR)begin PSLVERR <=1; end else begin PRDATA <= mem[PADDR[2:0]]; //read end end

In TB: should follow those specification

parameter int MAX_VALID_ADDR = 255; // pass to UVM world initial begin uvm_config_db#(int)::set(null, "*", "max_valid_addr", MAX_VALID_ADDR); end

In  Transaction:

int max_valid_addr; function void pre_randomize(); if (!uvm_config_db#(int)::get(null, "", "max_valid_addr", max_valid_addr)) `uvm_fatal("CFG", "max_valid_addr not set"); endfunction constraint invalid_addr { PADDR inside {[max_valid_addr : max_valid_addr + 155]}; }

2nd RUN ,  Simulation Results:

Functional Coverage:

| Metric | Covered | Total | % Hit | Status |
| --- | --- | --- | --- | --- |
| Overall Covergroup | 17 | 18 | 97.22% | ❌ Uncovered |

| Coverpoint Cp_write |  |  | Coverpoint cp_data |  |  |
| --- | --- | --- | --- | --- | --- |
| Bin | Hits | Status | BIn | Hits | Status |
| READ | 21 | ✅ Covered | Zero | 21 | ✅ Covered |
| WRITE | 19 | ✅ Covered | others | 19 | ✅ Covered |
| Coverpoints cp_addr |  |  | Coverpoint cp_error |  |  |
| Bin | Hits | status | Bin | Hits |  |
| Low (0-15) | 4 | ✅ Covered | no_error | 26 | ✅ Covered |
| Mid (16-127) | 14 | ✅ Covered | error | 14 | ✅ Covered |
| High(128-255) | 22 | ✅ Covered |  |  |  |

| Combination | Hits | Status |
| --- | --- | --- |
| READ, low | 0 | missing |
| WRITE, low | 4 | ✅ Covered |
| READ, mid | 8 | ✅ Covered |
| WRITE, mid | 6 | ✅ Covered |
| READ, high | 13 | ✅ Covered |
| WRITE, high | 9 | ✅ Covered |

| Combination | Hits | Status |
| --- | --- | --- |
| READ, no_error | 11 | ✅ Covered |
| WRITE, no_error | 15 | ✅ Covered |
| READ, error | 10 | ✅ Covered |
| WRITE, error | 4 | ✅ Covered |

Summary of coverage result:

No read transaction generated in low address range

READ × low address

✔ Error handling fully verified

Simulation Result:

Adding Assertions:

//`include "interface.sv" module apb_assertions(apb_inf inf); //default clock default clocking cb @(posedge inf.Pclk); endclocking // 1) setup- address phase property setup_to_address; disable iff (!inf.reset) (inf.PSEL && !inf.PENABLE) ¦=>(inf.PSEL && inf.PENABLE); endproperty ass_setup_to_address: assert property(setup_to_address); //else $error("APB: setup to address phase not followed"); //PENABLE only high when slave is selected property penable_implies_psel; disable iff (!inf.reset) (inf.PENABLE) ¦-> (inf.PSEL); endproperty ass_penable_implies_psel: assert property (penable_implies_psel); // 2) during wait state - PENABLE constant property p_wait_state_hold; disable iff (!inf.reset) (inf.PSEL && inf.PENABLE && !inf.PREADY) ¦=> (inf.PENABLE && inf.PSEL); endproperty ass_wait_state_hold: assert property(p_wait_state_hold); //PENABLE should drop after PREADY property p_complete_transfer; disable iff (!inf.reset) (inf.PSEL && inf.PENABLE && inf.PREADY) ¦=> (!inf.PENABLE); endproperty ass_complete_transfer : assert property (p_complete_transfer); //3 signal stability - //Address Control signals stable during PENABLE is high property p_ctrl_stable; disable iff (!inf.reset) (inf.PSEL && inf.PENABLE && !inf.PREADY) ¦-> $stable({inf.PADDR , inf.PWRITE}); endproperty ass_ctrl_stable : assert property (p_ctrl_stable); //PWDATA should be stable if PREADY = 0 property p_Wdata_stable; disable iff (!inf.reset) (inf.PSEL && inf.PENABLE && inf.PWRITE && !inf.PREADY) ¦-> $stable(inf.PWDATA); endproperty ass_Wdata_stable: assert property(p_Wdata_stable); //Read data valid when PREADY high property p_Rdata_valid; disable iff(!inf.reset) (inf.PSEL && inf.PENABLE && !inf.PWRITE && inf.PREADY) ¦-> !$isunknown(inf.PRDATA); endproperty ass_Rdata_valid: assert property(p_Rdata_valid); //4) basic rules of APB // when slave is not selected then PENABLE not asserted property p_idle_rule; disable iff (!inf.reset) (!inf.PSEL) ¦-> (!inf.PENABLE); endproperty ass_idle_rule:assert property(p_idle_rule); // when slave is selected then PADDR , PWRITE , PENABLE must not be X property p_ctrl_notunknown; disable iff (!inf.reset) inf.PSEL ¦-> !$isunknown({inf.PADDR , inf.PWRITE, inf.PENABLE}); endproperty ass_ctrl_notunknown: assert property(p_ctrl_notunknown); // 5) ERROR handling //no error during wait states property p_noerror_wait; disable iff(!inf.reset) (inf.PSEL && inf.PENABLE && !inf.PREADY) ¦-> (inf.PSLVERR == 0); endproperty ass_noerror_wait: assert property(p_noerror_wait); endmodule In TB: Bind apb_inf apb_assertions apb_assert_inst (inf);

Error encountered :

# Top level modules:

# 	tb

# End time: 22:51:18 on May 04,2026, Elapsed time: 0:00:00

# Errors: 0, Warnings: 0

# vsim -voptargs="+acc" tb "+UVM_TESTNAME=apb_test"

# Start time: 22:50:35 on May 04,2026

# ** Note: (vsim-3812) Design is being optimized...

# ** Error: tb.sv(14): (vopt-2907) Parameterizing 'inf' has exceeded the recursion depth limit of 10000.

# Optimization failed

Meaning:

The simulator got stuck in an infinite recursive reference involving inf (your interface)

I wrote: module apb_assertions(apb_inf inf);

Then bind:  bind apb_inf apb_assertions apb_assert_inst (inf);

When we bind apb_inf ..., the tool tries to:

- Attach apb_assertions to every instance of apb_inf

- But your module itself takes apb_inf inf as a port

- That creates recursive elaboration of interface inside itself

👉 Result:

interface → assertion → interface → assertion → ... (infinite)

Error resolving->  remove bind and instantiate assertion module in TB .

This avoids recursion because:

- You are not binding to interface type

- You are connecting actual instance

apb_assertions apb_assert_inst (inf);

Commands :

vsim -voptargs="+acc" -assertdebug tb

run -all

View Assertion Report:

We can see assertion errors in transcript and debug it by capturing exact time given in transcript and Inspect the waveform.